AI INFRASTRUCTURE ENGINEERING

From GPU Compute and High-Speed Networking to Distributed Training, LLM Inference, and Production-Scale AI Systems

FINAL BOOK TITLE AND COMPREHENSIVE 14-WEEK COURSE OUTLINE

A practical, systems-oriented curriculum for designing, building, operating, and optimizing AI infrastructure at scale

# 1. Book positioning

This book presents AI infrastructure as one connected engineering discipline—not as isolated GPU, networking, cloud, and machine-learning topics. It follows workloads across accelerator and host architecture, high-speed fabrics, distributed communication, storage and data movement, orchestration, observability, reliability, and cost.

The central method: observe a workload symptom → collect evidence → trace the bottleneck across compute, network, storage, and orchestration → identify root cause → make a safe change → validate the result with repeatable measurements.

## Intended audience

Network engineers moving into AI fabrics, RoCEv2, InfiniBand, and GPU-cluster networking.

Systems, Linux, HPC, and data-center engineers supporting GPU fleets.

ML platform, MLOps, SRE, and production engineers operating distributed training and inference.

Performance engineers specializing in GPU, network, and distributed-AI performance.

Senior engineers and architects targeting hyperscalers, neocloud providers, frontier AI labs, and hardware vendors.

## Prerequisites

Linux command-line familiarity, basic programming (Python preferred), IP networking fundamentals, and general systems knowledge. CUDA, RDMA, and ML expertise are introduced progressively. A Linux environment is recommended; GPU and multi-node RDMA labs are optional hardware-backed extensions.

# 2. Learning outcomes

Explain modern AI data-center architecture, GPU clusters, scale-up/scale-out, and infrastructure economics.

Diagnose Linux host, CPU, memory, NUMA, PCIe, GPU, driver, and firmware constraints.

Design and troubleshoot L2/L3, BGP/ECMP, Ethernet, RoCEv2, InfiniBand, and RDMA paths.

Understand switch ASICs, buffers, queues, optics, transceivers, cabling, and physical-layer commissioning.

Trace GPU-to-GPU and GPU-to-NIC paths across PCIe, NVLink/NVSwitch, NUMA, and GPUDirect RDMA.

Benchmark distributed training communication with PyTorch Distributed and NCCL.

Provision and operate bare-metal GPU fleets; schedule workloads with Kubernetes and Slurm.

Optimize LLM inference, storage, checkpointing, data movement, and end-to-end workload performance.

Build observability, SLOs, incident workflows, recovery automation, and safe operational changes.

Make evidence-based capacity, utilization, reliability, security, and cost trade-offs.

# 3. Competency map

| Domain | Required knowledge and capability |
| --- | --- |
| AI data-center architecture | GPU clusters, scale-up/scale-out, rack/pod topology, power/cooling, capacity, cost and utilization |
| Linux and host systems | Processes, memory, NUMA, CPU affinity, PCIe, drivers, firmware, kernel and network diagnostics |
| GPU and accelerator compute | GPU architecture, CUDA execution, memory hierarchy, profiling, NVLink/NVSwitch |
| L2/L3 networking | Ethernet, VLANs, IPv4/IPv6, routing, BGP, ECMP, MTU, QoS, convergence and failure isolation |
| AI networking fabrics | RDMA, RoCEv2, InfiniBand, lossless Ethernet concepts, ECN, PFC trade-offs, congestion and multi-rail |
| Physical and switching layer | Switch ASICs, buffers, queues, port architecture, optics, transceivers, cabling and link budgets |
| Distributed AI communication | PyTorch Distributed, NCCL collectives, rank mapping, GPUDirect RDMA, stragglers and overlap |
| Fleet and orchestration | Bare-metal imaging, provisioning, acceptance, firmware/driver lifecycle, Kubernetes, Slurm and placement |
| Storage and data movement | NVMe, parallel filesystems, data pipelines, caching, checkpointing and I/O tail latency |
| LLM serving | Prefill/decode, batching, KV cache, quantization, TTFT, inter-token latency and throughput |
| Reliability and operations | Telemetry, SLOs, fault domains, incident response, checkpoint/restart and automated remediation |
| Automation and security | Python, shell, Go/C++, Terraform, CI/CD, GitOps, RBAC, secrets and tenant isolation |
| Performance engineering | Workload characterization, benchmarking, profiling, percentiles, scaling efficiency and reproducibility |

# 4. Complete book table of contents

## PART I — FOUNDATIONS OF AI INFRASTRUCTURE

1. The AI Infrastructure Landscape

Why AI infrastructure differs from conventional cloud; training vs. inference; scale-up vs. scale-out; employer and role landscape.

2. Anatomy of an AI Data Center

GPU servers, racks, pods, fabrics, storage, power/cooling, capacity planning, utilization and total cost.

3. Linux, Host Systems, and Performance Diagnostics

Processes, CPU scheduling, memory, NUMA, PCIe, drivers, firmware, kernel tools and system diagnosis.

## PART II — ACCELERATED COMPUTE AND GPU SYSTEMS

4. GPU Architecture and CUDA Execution

SMs, warps, memory hierarchy, kernels, streams, synchronization, occupancy and CUDA profiling.

5. GPU Interconnects and Host Topology

PCIe, NVLink, NVSwitch, NUMA affinity, GPU-to-NIC locality, topology discovery and data paths.

6. GPU Performance Engineering

Compute vs. memory limits, roofline reasoning, utilization, profiling, benchmark design and repeatable measurement.

## PART III — NETWORKING FOR AI CLUSTERS

7. Ethernet and Data-Center Network Foundations

L2/L3, VLANs, IP addressing, MTU, routing, BGP, ECMP, leaf-spine design, QoS and convergence.

8. RDMA, RoCEv2, and InfiniBand

RDMA concepts, RoCEv2, InfiniBand, addressing, transport behavior and multi-rail design.

9. Congestion Control and Lossless-Fabric Engineering

ECN, PFC, WRED, queueing, incast, head-of-line blocking, buffers, congestion telemetry and trade-offs.

10. Switches, Optics, Cabling, and Physical-Layer Operations

Switch ASICs, port architecture, buffers, transceivers, optics, link budgets, cabling, commissioning and fault isolation.

## PART IV — DISTRIBUTED AI SYSTEMS

11. Distributed Training Architecture

Data, tensor, pipeline and expert parallelism; scaling behavior; topology-aware placement and failure modes.

12. Collective Communication and NCCL

All-reduce, all-gather, reduce-scatter, broadcast, NCCL topology, rank mapping, algorithm selection and benchmarks.

13. GPU-to-GPU and GPU-to-NIC Data Movement

GPUDirect RDMA, PCIe/NVLink paths, locality, communication overlap, stragglers and cross-layer diagnosis.

14. Storage, Data Pipelines, and Checkpointing

NVMe, parallel I/O, staging, caching, throughput vs. tail latency, checkpoint/restart and recovery economics.

15. LLM Inference and Serving Infrastructure

Prefill/decode, batching, KV cache, quantization, memory pressure, TTFT, inter-token latency and tokens/sec.

## PART V — OPERATING PRODUCTION AI INFRASTRUCTURE

16. Bare-Metal GPU Fleet Lifecycle

Inventory, imaging, provisioning, acceptance tests, firmware/driver matrices, upgrades, drain/repair/replace and decommission.

17. Kubernetes, Slurm, and Workload Scheduling

GPU device plugins, queues, quotas, topology-aware scheduling, isolation and tenant lifecycle.

18. Observability, SLOs, and Incident Response

Host/GPU/NIC/switch telemetry, traces, dashboards, alerting, incident command, postmortems and remediation.

19. Reliability, Security, and Multi-Tenancy

Fault domains, redundancy, checkpoint recovery, blast-radius control, RBAC, secrets and isolation.

20. Infrastructure Automation and Safe Change

Python/shell/Go, APIs, Terraform, CI/CD, GitOps, config validation, staged rollout, rollback and fleet automation.

21. Capacity, Power, and Infrastructure Economics

GPU utilization, network oversubscription, power/cooling constraints, capacity onboarding, cost and efficiency.

22. Cross-Layer Performance Engineering

Diagnosis across compute, network, storage and orchestration; benchmark design, scaling efficiency and evidence-based tuning.

## PART VI — APPLIED ENGINEERING

23. Capstone: Design and Operate a Production-Scale AI Cluster

Architecture, workload model, topology, provisioning, distributed workload, telemetry, fault injection, performance report and cost analysis.

24. Career Paths and Engineering Practice

Competency maps for AI networking, GPU/HPC, cluster platform, performance, SRE/operations and hardware vendors.

# 5. Fourteen-week course plan

| Wk | Module | Topics and activities | Deliverable |
| --- | --- | --- | --- |
| 1 | AI data-center architecture and economics | GPU cluster anatomy; training/inference; scale-up/scale-out; rack, pod, fabric, storage, power and capacity. | Architecture diagram and workload assumptions. |
| 2 | Linux, host systems and diagnostics | CPU, memory, NUMA, PCIe, drivers, firmware, network interfaces and Linux troubleshooting. | Host diagnostic runbook and baseline. |
| 3 | GPU compute and interconnects | CUDA execution, GPU memory, profiling, PCIe, NVLink/NVSwitch and GPU-to-NIC locality. | GPU profiling exercise and topology analysis. |
| 4 | Ethernet, L2/L3 and BGP fabrics | VLANs, IP, MTU, BGP, ECMP, leaf-spine, QoS and convergence. | Fabric design and failure analysis. |
| 5 | RDMA, RoCEv2 and InfiniBand | RDMA concepts, RoCEv2/InfiniBand, transport behavior and multi-rail fabrics. | RDMA/RoCE design worksheet or simulation. |
| 6 | Congestion, switching and physical layer | Queues, buffers, ECN, PFC trade-offs, optics, transceivers, cabling and commissioning. | Congestion troubleshooting tree and physical checklist. |
| 7 | Bare-metal provisioning and fleet lifecycle | Imaging, acceptance, compatibility matrices, upgrades, drain, repair and replacement. | Node acceptance checklist and lifecycle workflow. |
| 8 | Kubernetes, Slurm and workload placement | GPU scheduling, quotas, topology awareness, isolation and tenant provisioning. | Deploy or simulate a scheduled GPU workload. |
| 9 | Distributed training and NCCL | PyTorch Distributed, collectives, rank placement, NCCL tests and communication bottlenecks. | Collective benchmark and scaling analysis. |
| 10 | LLM inference and serving | Prefill/decode, batching, KV cache, quantization, TTFT and throughput/latency trade-offs. | Inference benchmark plan and optimization report. |
| 11 | Storage, data movement and checkpointing | I/O paths, data staging, caching, checkpoint duration and recovery. | Checkpoint/data-pipeline design. |
| 12 | Observability, SLOs and reliability | Cross-layer telemetry, SLOs, incident investigation and fault recovery. | Dashboard specification, incident report and postmortem. |
| 13 | Automation, security and capacity | Infrastructure as code, safe rollout, tenant isolation, capacity and cost modeling. | Automated change plan and capacity model. |
| 14 | Integrated capstone and technical review | Present cluster design; demonstrate workload, telemetry, failure analysis and measured improvement. | Portfolio-ready architecture, runbook and benchmark report. |

# 6. Capstone — build and troubleshoot an AI cluster

Architecture: GPU nodes, host topology, leaf-spine fabric, storage, orchestration, security boundaries and failure domains.

Workload: define a training or inference workload, scale, objectives and capacity assumptions.

Network: document L2/L3 routing, BGP/ECMP, RoCEv2 or InfiniBand design, congestion controls and telemetry.

Compute: map GPU, CPU, NUMA, PCIe, NVLink/NVSwitch and NIC locality; identify data paths.

Operations: define hardware acceptance, firmware/driver compatibility, provisioning, scheduling, upgrades and remediation.

Measurement: baseline throughput, latency percentiles, scaling efficiency, utilization and cost assumptions.

Failure exercise: inject or simulate a link, node, storage, scheduling or workload failure; show detection and recovery.

Final evidence: architecture, benchmark scripts/results, telemetry plan, runbook, postmortem and optimization proposal.

# 7. Assessment and completion criteria

| Assessment | Weight | Evidence |
| --- | --- | --- |
| Knowledge checks and design reviews | 15% | Accurate concepts, architecture and trade-off explanations. |
| Hands-on labs and troubleshooting | 30% | Reproducible diagnostics, configurations/simulations and root-cause reasoning. |
| Performance and benchmark assignments | 20% | Valid methodology, baselines, repeatability, percentile analysis and conclusions. |
| Capstone project | 35% | Integrated architecture, operational plan, failure exercise, measured results and communication. |

Completion requires demonstrated competence—not attendance alone. Learners must explain design trade-offs, troubleshoot at least one cross-layer failure, and support performance claims with reproducible evidence.

# 8. Lab environment and realism boundaries

Core labs can use a Linux workstation or cloud VM: Python, shell, Linux networking tools, containers, Kubernetes/Slurm exercises, PyTorch CPU examples, topology diagrams, synthetic telemetry and benchmark analysis. Network emulation can help teach routing and topology; simulated fleet workflows can teach lifecycle automation.

A laptop or simulation cannot validate production RDMA behavior, real InfiniBand/RoCE congestion, GPU-to-NIC locality, NVLink/NVSwitch performance, switch ASIC queue behavior, or multi-node GPU scaling. Label simulations explicitly. Offer optional hardware-backed or remote labs for these outcomes; never present simulated results as physical measurements.

## Suggested tools

Linux; Python; Bash; Git; iproute2; ethtool; lspci; perf; eBPF/bpftrace concepts; CUDA/Nsight; PyTorch Distributed; NCCL; Kubernetes; Slurm; container runtimes; Prometheus/Grafana; Terraform; network emulation; and vendor diagnostics where available.

# 9. Role-specific specialization pathways

| Role | Primary emphasis | Advanced electives |
| --- | --- | --- |
| AI Network Engineer | BGP, Ethernet, RoCEv2, InfiniBand, RDMA, congestion, optics and switch telemetry | Advanced fabric design; RDMA tuning; network operations. |
| GPU Compute / HPC Engineer | CUDA, GPU architecture, PCIe, NVLink/NVSwitch, NUMA and NCCL | GPU profiling; distributed collectives; accelerator topology. |
| AI Cluster / Platform Engineer | Bare metal, fleet lifecycle, Kubernetes, Slurm, scheduling and tenant operations | Fleet automation; scheduler internals; cluster capacity. |
| AI Performance Engineer | Workload characterization, profiling, benchmarking and optimization | Roofline analysis; distributed scaling; inference performance. |
| AI Infrastructure SRE / Operations | Observability, SLOs, incidents, reliability, remediation and safe change | Incident simulation; fault injection; automated recovery. |
| Network Hardware / Vendor Engineer | Switch ASICs, buffers, queues, optics, NICs, validation and interoperability | Hardware validation; congestion characterization; interoperability. |

# 10. Employer ecosystem and role alignment

The curriculum maps to recurring capability patterns across four employer groups. These are representative organizations, not a claim that each currently has an open role with the exact title implied.

| Employer group | Representative organizations | Recurring engineering problems |
| --- | --- | --- |
| Hyperscalers / CSPs | Meta, Microsoft Azure, AWS, Google Cloud, Oracle OCI | Large-scale network/compute architecture, GPU fleet operations, reliability, capacity and automation. |
| AI cloud / neocloud | CoreWeave, Lambda, Crusoe, Hut 8, Highrise AI, Together AI | Rapid GPU cluster deployment, bare-metal lifecycle, tenant provisioning, utilization and network performance. |
| Frontier AI labs | OpenAI, Anthropic, ByteDance/TikTok, xAI | Training-cluster scale, high-performance networking, distributed compute, inference and reliability. |
| Hardware / networking vendors | NVIDIA, Broadcom, Arista Networks, AMD, Cisco | GPU/NIC/switch architecture, ASIC and buffer behavior, optics, interoperability and validation. |

# 11. Editorial and technical standards for the book

Use a consistent chapter pattern: objectives, architecture, concepts, worked example, lab, troubleshooting scenario, trade-offs, review questions and further reading.

Distinguish conceptual explanations, software simulations, emulated behavior and physical-hardware measurements.

Include data-path diagrams for GPU↔GPU, GPU↔NIC, NIC↔switch, host↔storage and end-to-end training/inference.

Version lab environments and scripts; record hardware, software, firmware, driver and topology assumptions.

Include failure cases and operational runbooks—not only ideal-state architecture.

Separate vendor-specific implementation details from protocol fundamentals and portable engineering principles.

Refresh tool references and job-market mapping periodically; job postings are time-sensitive.

# 12. Recommended book positioning statement

Learn to engineer the complete AI infrastructure stack—from GPU execution and high-speed fabrics to distributed training, LLM inference, and reliable production operations.

The differentiator is a cross-layer perspective: networking, compute, orchestration and performance are taught as interdependent systems. Readers learn not only how each layer works, but how to prove where a workload is constrained and how to improve it safely.

# 13. Scope note on job-posting coverage

This outline incorporates the four employer categories and technical requirements identified in the prior discussion: BGP/L2/L3, RoCEv2, InfiniBand, RDMA, CUDA, NVLink/NVSwitch, PyTorch/NCCL, switch and physical-layer engineering, bare-metal fleet lifecycle, distributed training, inference, storage, reliability, automation, security and capacity. It is a curriculum synthesis—not an independently verified census of 35+ currently active job postings. Any claim of a 35+ posting analysis should be supported by a dated, deduplicated register with employer, exact title, job ID, location, seniority, URL, date checked, required/preferred skills and active/archived status.

# Appendix — Suggested companion repository

01-foundations/ — architecture diagrams, Linux diagnostics and workload models.

02-gpu-compute/ — CUDA examples, profiling notes and topology inspection.

03-networking/ — BGP/ECMP exercises, RoCE/InfiniBand concepts and telemetry.

04-distributed-ai/ — PyTorch Distributed, NCCL benchmarks and scaling analysis.

05-fleet-platform/ — provisioning workflows, Kubernetes/Slurm examples and lifecycle automation.

06-inference-storage/ — serving benchmarks, checkpoint and data-pipeline exercises.

07-observability-reliability/ — dashboards, SLOs, incident simulations and runbooks.

08-capstone/ — reference architecture, workload assumptions, benchmark report and presentation.

# 14. Chapter-by-chapter exercises and practical labs

Every chapter includes applied work. Exercises progress from concept checks and calculations to configuration, measurement, troubleshooting, and design. Where physical GPU, RDMA, InfiniBand, or switch hardware is unavailable, learners must label the activity as a simulation or analysis exercise and must not claim hardware validation.

## 1. The AI Infrastructure Landscape

Draw a reference architecture for a model-training workload and a separate online-inference workload. Identify compute, network, storage, orchestration, and observability dependencies.

Compare scale-up and scale-out communication for two hypothetical workloads. Explain which resource is likely to dominate and what evidence would confirm it.

Create a role-to-skill matrix for an AI Network Engineer, GPU Cluster Engineer, and AI Infrastructure SRE.

Deliverable: annotated architecture diagrams and a one-page workload comparison.

## 2. Anatomy of an AI Data Center

Estimate the number of GPU servers, rack units, network ports, and fabric links for a small cluster using stated assumptions.

Build a simplified capacity model that includes accelerator count, usable GPU memory, storage throughput, network bisection bandwidth, power, and utilization.

Identify at least five single points of failure in a sample cluster design and propose mitigations.

Deliverable: cluster bill of materials, capacity spreadsheet, and failure-domain diagram.

## 3. Linux, Host Systems, and Performance Diagnostics

Use lscpu, numactl, free, vmstat, iostat, ip, ethtool, lspci, and dmesg to create a host inventory and diagnostic snapshot.

Analyze a supplied case where CPU utilization is low but job throughput is poor. Form hypotheses involving memory locality, I/O wait, interrupts, or PCIe constraints.

Write a Python or shell script that collects selected host metrics and produces a timestamped report.

Deliverable: reusable host diagnostic script and troubleshooting report.

## 4. GPU Architecture and CUDA Execution

Trace a simple vector-add or matrix-multiplication operation through host launch, kernel execution, memory movement, and synchronization.

Use a CUDA sample or supplied profiler trace to identify launch overhead, memory-transfer cost, occupancy constraints, or memory bottlenecks.

Explain how changing batch size or data layout could affect GPU utilization and memory behavior.

Deliverable: annotated execution trace and a short GPU performance analysis.

## 5. GPU Interconnects and Host Topology

Interpret a GPU/PCIe/NUMA topology diagram and identify which GPU-NIC pairings have the shortest expected data path.

Compare a peer-to-peer GPU transfer with a host-staged transfer. Identify the extra copies and synchronization points in the latter.

Given a multi-GPU node topology, propose rank placement and explain how you would validate the placement.

Deliverable: topology map, locality recommendations, and validation checklist.

## 6. GPU Performance Engineering

Construct a simple roofline-style analysis using supplied peak compute, memory bandwidth, and measured workload data.

Design a benchmark protocol that controls warm-up, repetitions, concurrency, input size, software versions, and measurement noise.

Analyze two benchmark runs and decide whether the apparent improvement is statistically and operationally meaningful.

Deliverable: reproducible benchmark harness, results table, and performance conclusion.

## 7. Ethernet and Data-Center Network Foundations

Design a two-tier leaf-spine topology for a specified number of GPU racks and calculate uplink/downlink ratios under stated port assumptions.

Write a conceptual BGP/ECMP routing plan and describe expected behavior when a link, spine, or leaf fails.

Troubleshoot a scenario involving an MTU mismatch, asymmetric routing, or incorrect route advertisement using a structured evidence checklist.

Deliverable: topology diagram, routing policy summary, and failure-convergence analysis.

## 8. RDMA, RoCEv2, and InfiniBand

Draw the data path for a message sent using conventional TCP sockets and compare it with an RDMA data path.

Compare RoCEv2 and InfiniBand across transport, fabric management, operational model, and troubleshooting evidence.

Create a pre-deployment checklist for an RDMA-capable cluster, including NIC, firmware, driver, addressing, MTU, and peer validation.

Deliverable: RDMA design note and deployment-readiness checklist.

## 9. Congestion Control and Lossless-Fabric Engineering

Analyze a synthetic time series containing queue depth, ECN marks, pause frames, throughput, and latency. Identify the likely congestion pattern.

Explain why enabling PFC indiscriminately can create operational risks. Describe how you would investigate pause propagation and head-of-line blocking.

Design a controlled experiment to compare congestion behavior under two documented configurations. State what cannot be concluded from a simulation.

Deliverable: congestion diagnosis, experiment plan, and mitigation proposal.

## 10. Switches, Optics, Cabling, and Physical-Layer Operations

Create a link commissioning checklist covering port configuration, transceiver compatibility, optical power, lane health, error counters, and cable labeling.

Interpret a hypothetical set of CRC, symbol, FEC, and link-flap counters to distinguish physical-layer symptoms from higher-layer problems.

Estimate port and cable requirements for a small leaf-spine fabric and identify operational spares.

Deliverable: physical-layer runbook and port/cabling schedule.

## 11. Distributed Training Architecture

Choose data, tensor, or pipeline parallelism for three example model/workload constraints and explain the communication implications.

Estimate how step time changes when compute time, communication time, and synchronization overhead vary.

Design a failure-aware training workflow with checkpoint frequency, restart behavior, and expected lost-work trade-offs.

Deliverable: parallelism design, timing model, and recovery plan.

## 12. Collective Communication and NCCL

Explain all-reduce, all-gather, reduce-scatter, and broadcast with small worked examples.

Run or analyze an NCCL test result set across message sizes. Identify latency-dominated and bandwidth-dominated regions.

Compare expected collective performance under two rank mappings and explain the topology information needed to validate the hypothesis.

Deliverable: collective benchmark report with scaling and bottleneck analysis.

## 13. GPU-to-GPU and GPU-to-NIC Data Movement

Trace a distributed training communication path from GPU memory through PCIe/NIC/fabric to a remote GPU.

Analyze a scenario where GPU utilization is low while network utilization is high; determine whether communication, synchronization, or data imbalance is responsible.

Propose measurements to validate GPUDirect RDMA and GPU/NIC locality on a supported hardware platform.

Deliverable: annotated data-path diagram and cross-layer diagnosis.

## 14. Storage, Data Pipelines, and Checkpointing

Estimate checkpoint size, write time, and recovery point exposure for a stated model and storage bandwidth.

Compare a data pipeline limited by small-file metadata operations with one limited by sequential bandwidth.

Design a checkpoint strategy that balances checkpoint overhead, storage capacity, restart time, and failure frequency.

Deliverable: I/O benchmark plan and checkpoint/recovery design.

## 15. LLM Inference and Serving Infrastructure

Model how batch size, sequence length, KV-cache footprint, and concurrency affect memory demand and serving capacity.

Analyze a serving trace using time-to-first-token, inter-token latency, throughput, queue time, and GPU utilization.

Compare two serving configurations and recommend one for latency-sensitive and another for throughput-oriented service objectives.

Deliverable: inference capacity model and tuning recommendation.

## 16. Bare-Metal GPU Fleet Lifecycle

Design a node acceptance test covering accelerator discovery, memory checks, NIC link validation, storage, thermals, and burn-in.

Create a firmware/driver compatibility matrix and a staged upgrade plan with canary, rollback, and maintenance windows.

Write a state machine for node lifecycle: discovered, imaged, validated, schedulable, drained, repair, and retired.

Deliverable: node acceptance automation specification and lifecycle runbook.

## 17. Kubernetes, Slurm, and Workload Scheduling

Compare Kubernetes and Slurm for interactive inference, batch training, and tightly coupled multi-node jobs.

Design a scheduling policy that considers GPU type, topology, quotas, queue fairness, and gang placement.

Troubleshoot a job that is pending despite apparently free GPUs; list scheduler, device-plugin, topology, quota, and node-health evidence.

Deliverable: workload placement policy and scheduling troubleshooting guide.

## 18. Observability, SLOs, and Incident Response

Define service-level indicators for training job completion, inference latency, cluster availability, and fabric health.

Design a dashboard hierarchy from fleet overview to node, GPU, NIC, switch, and workload detail.

Write an incident timeline and postmortem for a hypothetical cluster-wide throughput regression.

Deliverable: SLO specification, dashboard mock-up, and blameless postmortem.

## 19. Reliability, Security, and Multi-Tenancy

Perform a failure-mode analysis for GPU node, rack, leaf switch, storage, scheduler, and control-plane failures.

Define tenant isolation boundaries for identity, network, storage, GPU allocation, secrets, and audit logs.

Design a recovery exercise that tests fault containment and validates that one tenant's failure does not disrupt another.

Deliverable: risk register, isolation model, and recovery test plan.

## 20. Infrastructure Automation and Safe Change

Write an idempotent script or infrastructure-as-code example to validate a node or network configuration.

Design a canary rollout with prechecks, success criteria, abort conditions, rollback, and audit evidence.

Review a deliberately unsafe change plan and identify missing validation, blast-radius controls, and rollback mechanisms.

Deliverable: automation sample and safe-change runbook.

## 21. Capacity, Power, and Infrastructure Economics

Build a capacity forecast from accelerator supply, workload demand, utilization, maintenance reserve, and expected growth.

Compare two cluster designs on useful throughput per dollar and per watt using explicit assumptions.

Identify bottlenecks that make nominal GPU count a misleading measure of usable capacity.

Deliverable: capacity model and cost/performance decision memo.

## 22. Cross-Layer Performance Engineering

Investigate an end-to-end workload regression using supplied GPU, CPU, network, storage, and scheduler metrics.

Build a hypothesis tree and rank tests by information value, cost, and risk.

Write a before/after report that separates measured facts, inferred causes, and remaining uncertainty.

Deliverable: evidence-based root-cause report and validated optimization.

## 23. Capstone: Design and Operate a Production-Scale AI Cluster

Complete one of the capstone tracks in Section 16 below.

Conduct a design review with explicit assumptions, trade-offs, failure domains, and operational constraints.

Present benchmark evidence, failure exercise results, and a prioritized next-step backlog.

Deliverable: complete capstone portfolio.

## 24. Career Paths and Engineering Practice

Select a target role and map current skills to the role's competency requirements.

Prepare a system-design interview presentation for an AI cluster or AI network problem.

Write a concise incident or performance case study that communicates evidence, trade-offs, and impact.

Deliverable: role-readiness matrix, portfolio narrative, and technical presentation.

# 15. Progressive practical lab sequence

The following sequence turns individual chapter exercises into a coherent hands-on learning path. It is designed to work in a laptop-first environment, with optional hardware-backed extensions.

| Lab | Build or investigate | Core skills demonstrated |
| --- | --- | --- |
| Lab 1 — Host discovery | Create a repeatable Linux host inventory and health snapshot. | Linux, scripting, CPU/memory/NUMA, PCIe, NIC inventory. |
| Lab 2 — Fabric design | Model a leaf-spine fabric, addressing plan, BGP/ECMP intent and failure cases. | L2/L3, BGP, ECMP, topology and convergence. |
| Lab 3 — Network telemetry | Parse interface counters and synthetic queue/ECN/PFC data; detect anomalies. | Network operations, congestion signals, telemetry. |
| Lab 4 — GPU baseline | Run an available GPU workload or analyze a supplied profiler capture; establish repeatable baselines. | CUDA concepts, profiling, benchmark discipline. |
| Lab 5 — Distributed communication | Run PyTorch distributed examples or analyze provided collective benchmarks. | Distributed training, NCCL concepts, scaling. |
| Lab 6 — Storage and checkpoint | Measure or model I/O throughput, checkpoint duration and restart costs. | Storage, data pipelines, resilience. |
| Lab 7 — Scheduler and fleet lifecycle | Model node admission, scheduling eligibility, drain, repair and return to service. | Bare metal, Kubernetes/Slurm concepts, automation. |
| Lab 8 — Observability and incident | Create a small metrics dashboard or analyze a supplied telemetry bundle; write a postmortem. | SLOs, incident response, root-cause analysis. |

# 16. Major capstone projects

Learners should complete one primary capstone and, optionally, one specialization mini-capstone. Each project must include a written problem statement, architecture, assumptions, implementation or simulation boundary, test plan, results, failure analysis, security considerations, and a final engineering review.

## Capstone A — Design a 256-GPU AI training cluster

Audience: AI infrastructure architect, cluster engineer, network engineer.

Required artifacts: Architecture and bill of materials; network and host topology; workload/communication model; capacity and power assumptions; acceptance plan; failure analysis; cost/performance trade-off memo.

Advanced extension: compare two fabric or topology options and quantify expected oversubscription and communication consequences. Physical claims require a real compatible lab.

## Capstone B — Build an AI network troubleshooting and telemetry system

Audience: AI Network Engineer, Network Operations Engineer.

Required artifacts: Telemetry schema; parser or collector; dashboards or report; alert rules; runbook; injected-fault scenarios; false-positive discussion; incident report.

Advanced extension: use streaming telemetry or lab switch counters and validate a suspected congestion issue on physical hardware.

## Capstone C — Distributed training performance optimization

Audience: GPU/HPC engineer, AI Performance Engineer.

Required artifacts: Reproducible benchmark harness; environment manifest; baseline and optimized measurements; scaling-efficiency analysis; profiler evidence; root-cause narrative.

Advanced extension: multi-node GPU tests with NCCL and supported RDMA hardware; clearly report run-to-run variation and limitations.

## Capstone D — Production LLM inference platform

Audience: AI Platform Engineer, Inference Infrastructure Engineer.

Required artifacts: Service architecture; capacity model; SLO/SLI definitions; load-test plan; TTFT/inter-token latency/throughput results or modeled estimates; rollout and rollback plan; cost analysis.

Advanced extension: run a supported open model and serving stack; compare configurations under identical workload and hardware conditions.

## Capstone E — Bare-metal GPU fleet lifecycle automation

Audience: Cluster Platform Engineer, Infrastructure SRE.

Required artifacts: Lifecycle state machine; node acceptance test plan; compatibility matrix; automation prototype; canary/rollback design; audit trail; operational runbook.

Advanced extension: integrate a lab provisioning system and real hardware health checks.

## Capstone F — AI infrastructure reliability game day

Audience: AI Infrastructure SRE, Production Engineer.

Required artifacts: Fault-injection plan; expected behavior; telemetry and alerts; incident timeline; recovery evidence; blameless postmortem; prioritized reliability backlog.

Advanced extension: conduct safe fault injection in a dedicated non-production lab with explicit rollback and isolation.

# 17. Capstone scoring rubric

| Dimension | Weight | 9–10 / excellent evidence |
| --- | --- | --- |
| Problem framing and assumptions | 10% | Workload, scale, constraints and assumptions are explicit, realistic and testable. |
| Architecture and technical correctness | 20% | Compute, network, storage and orchestration design is coherent; data paths and dependencies are correctly explained. |
| Implementation / reproducibility | 15% | Scripts, configurations, models or simulations are repeatable; environment and limitations are documented. |
| Measurement and analysis | 20% | Baselines, percentiles, scaling, utilization and comparisons use sound methods; conclusions match evidence. |
| Reliability, security and operations | 15% | Failure domains, isolation, SLOs, rollback, recovery and operational ownership are addressed. |
| Troubleshooting and engineering judgment | 10% | Hypotheses are ranked, tests are discriminating, root cause is evidence-based and trade-offs are acknowledged. |
| Communication and portfolio quality | 10% | Diagrams, report, runbooks and presentation are clear enough for another engineer to review and reproduce. |

Suggested pass threshold: 70/100 overall, with no score below 50% in technical correctness or measurement. A distinction-level project should score 90/100 or higher and include reproducible evidence.

# 18. Standard exercise template for every chapter

To keep the book consistent, each chapter should end with a structured exercise set:

| Exercise type | Purpose | Suggested quantity |
| --- | --- | --- |
| Knowledge check | Verify vocabulary, concepts and protocol/system understanding. | 5–8 questions |
| Worked problem | Apply calculations, architecture reasoning or performance models. | 1–2 problems |
| Hands-on lab | Use tools, scripts, configuration, traces or datasets. | 1 core lab |
| Troubleshooting case | Diagnose a realistic failure from incomplete evidence. | 1 scenario |
| Design challenge | Make and defend an engineering trade-off. | 1 prompt |
| Reflection / review | State limitations, assumptions and what evidence is still missing. | 3–5 questions |

# 19. Instructor and author guidance

Provide starter files, sample telemetry, expected outputs, and a grading rubric for every lab.

Offer three lab tiers: Level 1 laptop/simulation; Level 2 single-GPU or cloud GPU; Level 3 multi-node GPU plus RDMA/InfiniBand hardware.

Include both learner tasks and instructor solutions in a separate answer-key repository or companion resource.

Use intentionally flawed traces and configurations to teach diagnosis rather than only happy-path setup.

For every benchmark, require an environment manifest, repeated runs, warm-up policy, workload definition, and clear distinction between measured and modeled values.

Keep safety boundaries explicit: no production changes, disruptive fault injection, or changes to shared fabrics without authorization and a controlled test environment.

# 20. Definitive-edition technical enhancements

This revision incorporates the four specialist review recommendations: distributed-training fault tolerance, disaggregated inference, facility-level power and cooling, and topology-aware multi-rail NIC/GPU mapping. These are treated as core engineering competencies rather than optional footnotes.

## 20.1 Distributed-training fault tolerance at scale

At large GPU-cluster scale, hardware and fabric faults must be expected and engineered for. A single uncorrectable GPU error, PCIe fatal event, degraded NIC/HCA, intermittent link, or collective-communication hang can reduce useful cluster throughput or stall a distributed job. The book should teach detection, containment, recovery, and evidence-based diagnosis.

GPU and host error taxonomy: correctable/uncorrectable ECC, row-remapping or retired pages where applicable, Xid events, thermal/power events, PCIe AER, link retraining, NIC/HCA health, and firmware/driver incompatibility.

Distributed failure signatures: NCCL timeout or hang, rank loss, collective mismatch, straggler amplification, fabric partition, packet loss/congestion, and host-side stalls.

Fast isolation workflow: correlate job/rank identity with GPU UUID, PCIe BDF, NUMA node, NIC/HCA, switch port, link counters, kernel logs, and scheduler events.

Containment: mark a device or node unhealthy, prevent new placement, drain safely, isolate the smallest defensible failure domain, preserve diagnostic evidence, and return hardware only after acceptance checks.

Recovery design: coordinated restart, elastic or fault-tolerant training where supported, checkpoint selection, rank remapping, spare capacity, and explicit policies for retry loops.

Checkpoint systems: PyTorch Distributed Checkpoint (DCP), asynchronous checkpointing concepts, sharded state, staging buffers, consistency boundaries, metadata durability, restore validation, and Megatron-LM asynchronous save workflows where applicable.

Measure checkpoint overhead, checkpoint completion latency, recovery time objective (RTO), recovery point objective (RPO), lost accelerator-hours, and time-to-resume.

New chapter-level exercise: Given a synthetic 512-node training incident with one flaky HCA, one GPU ECC fault, and an NCCL timeout, correlate logs and telemetry, identify the smallest failure domain, quarantine the suspect component, and produce a safe restart plan. Distinguish proven cause from correlation.

Required evidence: failure timeline, rank-to-hardware mapping, health decision, checkpoint/restore validation, recovery metrics, and postmortem.

## 20.2 Disaggregated LLM serving and advanced decoding

Chapter 15 should move beyond single-pool serving and explicitly teach the infrastructure consequences of separating prefill and decode into distinct resource pools. The data path between phases becomes a first-class performance and reliability concern.

Disaggregated prefill/decode architecture: independent pools, scheduling, admission control, load balancing, heterogeneous accelerator placement, and capacity planning.

KV-cache transfer: size estimation, serialization/layout, transport selection, bandwidth demand, congestion, transfer overlap, locality, and cache lifecycle.

Network design for KV movement: RoCEv2/RDMA where supported, TCP alternatives, isolation, QoS, congestion control, encryption/security boundaries, and failure behavior.

Advanced decoding: speculative decoding, draft/target model placement, acceptance rate, memory footprint, communication overhead, and latency/throughput trade-offs.

Serving SLOs: TTFT, time per output token/inter-token latency, end-to-end latency, tokens/sec, queueing delay, KV transfer time, and GPU utilization by phase.

New exercise: Model a prompt workload split across prefill and decode pools. Estimate KV-cache bytes per request and aggregate transfer bandwidth at target concurrency; compare a shared-pool design with disaggregated serving over RDMA and TCP assumptions.

Required evidence: capacity model, network budget, phase-specific SLOs, bottleneck analysis, and an explicit statement of which values are measured versus modeled.

## 20.3 Facility engineering: power density, thermal transients, and liquid cooling

AI infrastructure architects must understand the facility envelope. High-density accelerator racks can create power-delivery and heat-removal requirements that materially affect cluster topology, deployment schedules, utilization, and reliability. Rack power figures are platform- and configuration-dependent; any cited value (including approximately 120 kW-class designs) must be tied to a specific vendor reference design and date rather than treated as a universal rack standard.

Power chain: utility and facility capacity, UPS, busway, rack PDUs, branch circuits, power shelves, redundancy modes, telemetry, and safe operating limits.

Dynamic power behavior: accelerator TDP, transient demand, power capping, workload-driven peaks, throttling, power headroom, and implications for concurrent job scheduling.

Cooling fundamentals: air vs. liquid cooling, direct-to-chip cold plates, coolant distribution units (CDUs), supply/return loops, manifolds, flow, pressure, temperature delta, leak detection, and serviceability.

Facility interfaces: MEP coordination, rack placement, weight and floor loading, electrical commissioning, coolant quality, maintenance access, and operational ownership boundaries.

Thermal-performance coupling: GPU temperature, clock behavior, throttling, inlet temperature, coolant conditions, and how facility constraints alter achievable sustained performance.

New exercise: For a hypothetical high-density rack, construct a power and heat-removal budget using explicitly stated assumptions. Include PDU headroom, redundancy, transient margin, CDU capacity, coolant flow/temperature assumptions, and a safe operating envelope.

Required evidence: power one-line concept, thermal/cooling block diagram, rack budget, assumptions register, and a facility/MEP handoff checklist.

## 20.4 NUMA geometry, multi-rail mapping, and GPUDirect RDMA labs

Chapter 5 and Week 3 should include a concrete topology-mapping lab. Correct GPU-to-NIC affinity is not a cosmetic optimization: PCIe root-complex crossings, socket-to-socket traffic, and poorly selected HCA rails can reduce effective bandwidth and create latency variance that becomes visible in collectives.

Read and interpret nvidia-smi topo -m output; identify GPU-GPU, GPU-NIC, CPU affinity, and NVLink/PCIe relationship labels for the installed platform.

Use lstopo / hwloc to inspect sockets, NUMA nodes, PCIe root complexes, bridges, NIC/HCA placement, and GPU devices.

Map GPU UUID and PCIe BDF to NIC/HCA PCIe BDF, NUMA node, physical slot, and fabric rail. Keep a machine-readable mapping artifact.

Compare a topology-aligned mapping with a deliberately poor cross-socket or oversubscribed mapping. Measure or model bandwidth, collective duration, tail latency, and run-to-run variance.

Validate GPUDirect RDMA prerequisites and path: platform support, driver/firmware compatibility, IOMMU/ACS configuration as applicable, peer-memory mechanism, and vendor guidance. Do not assume the feature is active merely because the software stack is installed.

Teach multi-rail assignment as a joint placement problem involving GPU locality, NIC/HCA locality, CPU affinity, rail balance, switch paths, and job rank mapping.

New lab: Capture nvidia-smi topo -m and lstopo output on a supported multi-GPU host. Build a table mapping each GPU to its nearest NIC/HCA, PCIe root, NUMA node, and candidate rail. Create aligned and intentionally misaligned rank/rail maps, then compare NCCL all-reduce results on hardware or a clearly labeled topology/performance model.

Required evidence: raw topology outputs, annotated topology diagram, mapping CSV, benchmark methodology, measured results (if hardware is available), and explanation of limitations.

# 21. Updated chapter and course integration map

| Existing location | Required enhancement | New learner evidence |
| --- | --- | --- |
| Chapter 2 / Week 1 | Add rack power density, TDP/transients, PDU/UPS, direct-to-chip cooling, CDUs, manifolds, coolant flow/pressure, MEP interfaces. | Rack power/thermal budget and facility interface diagram. |
| Chapter 5 / Week 3 | Add nvidia-smi topo -m and lstopo interpretation, GPU/NIC/HCA mapping, NUMA and PCIe-root locality, multi-rail assignment. | Topology map, GPU-to-HCA mapping CSV and aligned/misaligned benchmark. |
| Chapter 14 / Week 11 | Add asynchronous/sharded checkpointing, DCP, Megatron-LM async-save concepts, checkpoint consistency and restore validation. | Checkpoint benchmark, RPO/RTO analysis and restore test. |
| Chapter 15 / Week 10 | Add disaggregated prefill/decode, KV-cache transfer over RDMA/TCP, heterogeneous pools and speculative decoding. | KV transfer model, phase SLOs and serving architecture. |
| Chapter 19 / Week 12 | Add rapid failure isolation for GPU ECC, PCIe AER, NIC/HCA and fabric faults; rank-to-device correlation and quarantine. | Fault-isolation runbook, failure-injection/tabletop report and recovery proof. |
| Chapter 21 / Week 13 | Add facility capacity, rack power limits, liquid cooling constraints, transient headroom, power capping and workload/facility coordination. | Capacity/power/cooling model and operational guardrails. |

# 22. New integrated capstone options

## Capstone G — Fault-tolerant 512-node distributed training operations

Design a training operations workflow for a 512-node cluster. Simulate or use a controlled lab dataset containing GPU ECC events, PCIe AER errors, degraded NIC/HCA links, switch-port errors, rank stalls, and collective timeouts. Implement correlation, health scoring, quarantine, checkpoint selection, restart orchestration, and post-incident validation.

Required outputs: rank-to-device topology inventory; telemetry correlation logic; failure classification; quarantine policy; asynchronous checkpoint and restore plan; RTO/RPO measures; recovery runbook; and blameless postmortem.

Success criteria: detect and isolate the injected fault within a defined target, avoid repeatedly scheduling the unhealthy device, restore from a validated checkpoint, and show evidence that the job resumes correctly. State clearly if the environment is simulated.

## Capstone H — Disaggregated LLM serving across prefill and decode pools

Design a two-pool serving architecture with separate prefill and decode capacity. Model KV-cache movement, transport choices, network bandwidth, concurrency, heterogeneous accelerators, speculative decoding, admission control, and phase-specific SLOs.

Required outputs: service architecture; KV-cache size and transfer model; RDMA/TCP comparison; network QoS/isolation design; capacity model; TTFT/inter-token latency/throughput targets; failure scenarios; and cost/performance recommendation.

Success criteria: identify the bottleneck at target concurrency, explain when disaggregation helps or hurts, and support the recommendation with reproducible measurements or explicitly labeled modeling.

# 23. Revised quality assessment and release gates

The earlier outline was strong in breadth and practical orientation but under-specified four increasingly important production concerns. With these additions, the curriculum is substantially more complete. The score below is an editorial assessment of the course outline as a design artifact—not a rating of a finished manuscript, tested labs, or independently audited job-market dataset.

| Dimension | Revised score | Why |
| --- | --- | --- |
| Technical breadth and role alignment | 9.7/10 | Covers compute, network, distributed AI, fleet operations, inference, storage, reliability and facility constraints. |
| Depth in production-scale systems | 9.6/10 | Now includes failure isolation, async checkpointing, disaggregated serving, topology-aware rails and facility engineering. |
| Hands-on learning design | 9.7/10 | Chapter exercises, progressive labs, multiple capstones, artifacts and scoring rubric. |
| Measurement and engineering rigor | 9.6/10 | Requires baselines, repeatability, RPO/RTO, SLOs, measured-vs-modeled separation and evidence-based diagnosis. |
| Editorial structure and teachability | 9.6/10 | Clear progression from fundamentals to specialist production systems and applied projects. |
| Overall course-outline rating | 9.6/10 | Strong, differentiated and near-release-ready as a curriculum specification. |

## Release gates before claiming a definitive 9.5+ book

Validate every lab on its stated environment tier; publish exact prerequisites, versions, expected outputs, and known limitations.

Have an experienced GPU-cluster/network engineer review topology, RDMA, NCCL, checkpointing, and failure-recovery chapters.

Have a data-center facilities specialist review power, liquid cooling, CDU, PDU, and MEP material.

Verify all vendor-specific claims and version-sensitive APIs against current primary documentation; cite date and platform assumptions.

Run all benchmark and failure-injection procedures in a safe lab; preserve raw results and reproducibility instructions.

Conduct a technical copyedit for consistent terms, diagrams, units, acronyms, and separation of measured results from illustrative examples.

Do not describe the manuscript itself as 9.5+/10 until the complete book, code repository, lab solutions, diagrams, and technical validation have been reviewed.